Tutorial Sheet

- An insulating material 3 cm thick and 300 cm2 in area is to be heated by dielectric heating. The material has permittivity of 5 and power factor 0.05. Power required is 450 watts and frequency of 45 MHz is to be used. Determine the voltage necessary and the current that will flow through the material. If the voltage were to be limited to 900 V, will be the frequency to get the same loss.

- A 400 tone train travels down a gradient 1 in 70 for 120 sec. during which period its speed is reduced from 80 km/hr to 50 km/hr by regenerative breaking. Find the energy returned to the lines if tractive resistance is 5 kg/tone and allowance for rotational inertia is 7.5%. Overall efficiency of motors is 75%.

- Assuming a simplified speed time curve for a train for a train which has got a scheduled speed of 50 km/hr between two stops 2 km apart. Find the crest speed of the train over the run, if the stop is of 30 seconds duration and the values of acceleration and retardation are 2.5 km/hr/sec and 3.4 km/hr/sec.

- A room of 12 x 12 x 4 is to have direct lighting, giving illumination of 80 Lux, on a working plane 70 cm above the floor. Coefficient of utilization is 0.5 and maintenance factor 0.8. If efficiency of lamps available is 14.75 lumens/watt, find the number of lamps and their rating.

- A 30 KW, 3 phase, 400 V resistance oven is to employ nickel chromium strip 0.254 mm thick for the three star connected heating elements. If the wire temperature is to be 1100*C and that of the charge to be 700*C. Estimate a suitable width for the strip. Assume emissivity =0.9 and radiating efficiency to be 0.5. What would be the temperature of the wire if the charge were cold?

- A 300 tones E.M.U. train is started with a uniform acceleration and reaches a speed of 40km.p.h. in 24 sec. on level section. Find the specific energy consumption, assuming a simplified trapezoidal curve, with rotational inertia as 8% retardation as 3 km.p.h.p.s., the distance between two stations as 3 km, efficiency of motor as 0.9 and train resistance 5 kg/ tone.

- Two lamps of 500 W, each with lamp efficiency of 25 lumens per watt are mounted on two lamp post 10 m apart. The posts have different heights of 3 m and 4 m respectively. Calculate the illumination at a point mid way between the lamp posts.

- A train is required to run between two stations 3 km apart at an average speed of 50 kmph. The run is to be made to a simplified quadrilateral speed time curve. If the maximum speed is to be limited to 72 kmph, acceleration to 2 kmphps and coasting & breaking retardation to 0.16 kmphps and 3.2 kmphps respectively, determine the duration of accelerating, coasting and braking periods.

- A 3 phase arc furnace has to melt 10 tons of steel in 2 hrs. Determine the average kW input to the furnace if its overall efficiency is 50%. If the current input is 9000A with the above kW input and the resistance and reactance of the furnace are 0.003 ohms and 0.005 ohms respectively, determine the arc voltage and the total KVA taken from the supply. Assume latent heat of steel 0.12, latent heat of fusion of steel = 8.89 kcal/kg and melting point of steel = 1371*C.