Question Bank

- What are the advantages & disadvantages of Electric Traction System?

- Explain the following terms used in illumination:- (i) Luminous Flux (ii) Luminous Efficiency (iii) Luminous Intensity (iv) Lumen (v) Candle Power

- Define principle of Arc welding. What are the conditions for successful welding?

- What are the advantages of electric heating over other methods of heating?

- Explain Regenerative breaking used in traction system.

- Draw the construction diagram of Mercury Vapour Lamp & explain its working principal.

- Draw block diagram of an AC locomotive & explain its different components.

- Explain the following terms used in illumination:- (i) MSCP, MHCP & MHSCP (ii) Lamp Efficiency (iii) Depreciation Factor (iv) Co-efficient of Utilization (v) Waste Light Factor.

- Write short notes on Dielectric Heating.

- Explain different types of Resistance welding.

- Write short notes on Incandescent Bulb.

- Compare between AC & DC welding.

- Draw & explain Speed time Curve for Urban, Sub-Urban & Main Line Services.

- Explain the following terms used in illumination:- (i) Reflection Factor (ii) Reduction Factor (iii) Space Height Ratio (iv) Depreciation Factor (v) Absorption Factor.

- Define Laws of Illumination.

- Explain Dielectric Heating & mention their applications?

- Write short notes on Laser welding.

- Define Polar curve. How polar curve is used to determine MHCP & MSCP?

- Write short notes on followings: (i) Ideal Traction System (ii) Crest Speed (iii) Average speed (iv) Schedule speed.

- What is tractive effort? Derive expression for tractive effort and hence for power output from driving axles.

- Explain the working principal of Fluorescent Tube. What is the function of Starter & Choke in fluorescent tube?

- Define Straight & Reverse Polarity in DC welding.

- Write short note on Ajax-Wyatt Vertical Core Type Furnace.

- Write short notes on followings: (i) Specific Energy Consumption (ii) Accelerating Weight (iii) Co-efficient of adhesion (iv) Conductor Rail System.

- Write short notes on (i) Sodium Vapour Lamp (ii) Neon lamps & tubes.

- Explain different systems of traction.

- Write short notes on followings (i) Electrical Properties of an arc (ii) Drooping Characteristic for welding (iii) Types of Electrical Wiring.

- Explain coreless induction furnace.

- What are the different types of Lighting Schemes?

- What are the advantages & disadvantages of Electric Breaking? Explain Plugging for DC series motor.

- Compare between Arc Welding & Resistance welding.

- Discuss application of Electric Arc welding.

- Explain different types of Resistance Heating.

- An insulating material 3 cm thick and 300 cm2 in area is to be heated by dielectric heating. The material has permittivity of 5 and power factor 0.05. Power required is 450 watts and frequency of 45 MHz is to be used. Determine the voltage necessary and the current that will flow through the material. If the voltage were to be limited to 900 V, will be the frequency to get the same loss.

- A 400 tone train travels down a gradient 1 in 70 for 120 sec. during which period its speed is reduced from 80 km/hr to 50 km/hr by regenerative breaking. Find the energy returned to the lines if tractive resistance is 5 kg/tone and allowance for rotational inertia is 7.5%. Overall efficiency of motors is 75%.

- Assuming a simplified speed time curve for a train for a train which has got a scheduled speed of 50 km/hr between two stops 2 km apart. Find the crest speed of the train over the run, if the stop is of 30 seconds duration and the values of acceleration and retardation are 2.5 km/hr/sec and 3.4 km/hr/sec.

- A room of 12 x 12 x 4 is to have direct lighting, giving illumination of 80 Lux, on a working plane 70 cm above the floor. Coefficient of utilization is 0.5 and maintenance factor 0.8. If efficiency of lamps available is 14.75 lumens/watt, find the number of lamps and their rating.

- A 30 KW, 3 phase, 400 V resistance oven is to employ nickel chromium strip 0.254 mm thick for the three star connected heating elements. If the wire temperature is to be 1100*C and that of the charge to be 700*C. Estimate a suitable width for the strip. Assume emissivity =0.9 and radiating efficiency to be 0.5. What would be the temperature of the wire if the charge were cold?

- A 300 tones E.M.U. train is started with a uniform acceleration and reaches a speed of 40km.p.h. in 24 sec. on level section. Find the specific energy consumption, assuming a simplified trapezoidal curve, with rotational inertia as 8% retardation as 3 km.p.h.p.s., the distance between two stations as 3 km, efficiency of motor as 0.9 and train resistance 5 kg/ tone.

- Two lamps of 500 W, each with lamp efficiency of 25 lumens per watt are mounted on two lamp post 10 m apart. The posts have different heights of 3 m and 4 m respectively. Calculate the illumination at a point mid way between the lamp posts.

- A train is required to run between two stations 3 km apart at an average speed of 50 kmph. The run is to be made to a simplified quadrilateral speed time curve. If the maximum speed is to be limited to 72 kmph, acceleration to 2 kmphps and coasting & breaking retardation to 0.16 kmphps and 3.2 kmphps respectively, determine the duration of accelerating, coasting and braking periods.

- A 3 phase arc furnace has to melt 10 tons of steel in 2 hrs. Determine the average kW input to the furnace if its overall efficiency is 50%. If the current input is 9000A with the above kW input and the resistance and reactance of the furnace are 0.003 ohms and 0.005 ohms respectively, determine the arc voltage and the total KVA taken from the supply. Assume latent heat of steel 0.12, latent heat of fusion of steel = 8.89 kcal/kg and melting point of steel = 1371*C.